# School Accessibility Checklist.

| Name of the school: | Date of assessment: |
| --- | --- |
| Focal person / main school contact: |  |

| Enrolment data |  |  |  |
| --- | --- | --- | --- |
| Total number of students | Girls | Boys | Total |
| Total number of students with disabilities | Girls | Boys | Total |

| Building layout |  |  |
| --- | --- | --- |
| How many classrooms does the school have |  |  |
| How many classrooms are accessible to wheelchair users / students with reduced mobility? |  |  |
| How many floor levels does the school have |  |  |
| How many floor levels are accessible to wheelchair users / students with reduced mobility (ramps / lifts) |  |  |
| Teaching capacity |  |  |  |
| Total number of specialist staff trained to teach students with disabilities |  |  |  |
| Does the school have a resource room staffed with people trained on supporting students with disabilities | Yes ☐ | No ☐ |  |
| In the past year, how many teaching staff received in-service training on teaching students with disabilities |  |  |  |

| School Accessibility Checklist |  |  |  |  |
| --- | --- | --- | --- | --- |
| 01 | Entry | Yes | No | N/A |
|  | Are you able to identify the school buildings from signs outside of the school campus |  |  |  |
|  | Do signs have text which is a contrasting colour to the background e.g. white on blue or black on yellow |  |  |  |
|  | Can the sign be read easily from a reasonable distance including by children and young adults |  |  |  |
|  | Is there a working pedestrian crossing point known to be safe for children, to cross the road |  |  |  |
|  | Are kerb ramps provided at all levels between the road and the footpath leading into the school |  |  |  |
|  | If yes, are they wide enough to accommodate a wheelchair |  |  |  |
|  | Is main entrance to the school free of obstructions such as tree roots, open drains, uncovered ditches, parked cars |  |  |  |
|  | Are there steps leading up to the main entrance / administration block |  |  |  |
|  | If yes, is there a regulation ramp in good condition useable by a wheelchair-user |  |  |  |
|  | Are there levelled, smooth, continuous and non-slip surfaces taking you to classrooms / classroom blocks |  |  |  |
| Overall comments: |  |  |  |  |
| 02 | Ramps | Yes | No | N/A |
|  | Are there ramps next to steps and/or stairs |  |  |  |
|  | Are the location of the ramps clearly identifiable with a sign |  |  |  |
|  | Are ramps well-maintained & free of any obstructions |  |  |  |
|  | Are ramps at least the same width as the path or the corridor they connect to |  |  |  |
|  | Are ramps gradient no steeper than 1:12 |  |  |  |
|  | Are there continuous handrails, on both sides, at a height of 760mm-900mm from the floor surface |  |  |  |
|  | For ramps used by children, are the tops of secondary rails at a min. height of 600mm – 750mm from the floor surface |  |  |  |
|  | Are the ramps at least 1500mm wide between handrails |  |  |  |
|  | Do handrails extend 300mm from the end of ramps at top and bottom? They should not project more than 89mm beyond ramp or stairs |  |  |  |
|  | Are there landings of 1500mm x 1500mm at the turning point of the ramps |  |  |  |
|  | Do ramps have an anti-slip / matte surface |  |  |  |
|  | Is there protection at each edge to prevent wheelchairs from falling off |  |  |  |
| Overall comments: |  |  |  |  |
| 03 | Steps and stairways | Yes | No | N/A |
|  | Is the location of the stairs easily identifiable with a sign |  |  |  |
|  | Is the width of the stairs a minimum 1200mm |  |  |  |
|  | Are stair risers / height a minimum 100mm to 180mm with tread at minimum 280mm |  |  |  |
|  | Do step edges have a colour contrast strip or are they a different texture |  |  |  |
|  | Confirm there are no gaps between stair risers |  |  |  |
|  | Are there continuous handrails, on both sides, at a height of 760mm - 900mm from floor surface |  |  |  |
|  | Are handrails painted in contrasting colours to the walls |  |  |  |
| Overall comments: |  |  |  |  |
| 04 | Corridors | Yes | No | N/A |
|  | Is the minimum unobstructed width of corridors at least 1200mm |  |  |  |
|  | Is it true that there are no objects protruding more than 100mm from walls in corridors |  |  |  |
|  | Are all overhanging obstructions mounted above a minimum height of 2200mm from the floor surface |  |  |  |
| Overall comments: |  |  |  |  |
| 05 | Signage | Yes | No | N/A |
|  | Are there signs to indicate the floor level on all floors |  |  |  |
|  | Are signs placed at a height of 1000mm – 1600mm above the floor surface |  |  |  |
|  | Can you easily follow signs to get to different classrooms and other important places around the school |  |  |  |
|  | Are easy to follow pictograms or other visual information provided to support written directions |  |  |  |
|  | Are there signs for girls’ and boys’ general toilets |  |  |  |
|  | Are there signs for girls’ and boys’ accessible toilets |  |  |  |
|  | Can you follow signs to reach appropriate toilets with ease |  |  |  |
| Overall comments: |  |  |  |  |
| 06 | Doors | Yes | No | N/A |
|  | Is it possible for a child to open / close the door with ease and without the help of an adult or another child |  |  |  |
|  | If the doors have glass, is there a coloured strip (or other marking) placed at eye level |  |  |  |
|  | Are doorframes painted in contrasting colour to the walls |  |  |  |
|  | Are handles (locks, pulls) located at a height of 800-1000mm from the surface of the floor |  |  |  |
|  | Are door sills (thresholds) no more than 10mm high and bevelled (merged with a gentle slope) |  |  |  |
|  | Are doormats flushed / embedded in the flooring and edges secured to the floor |  |  |  |
| Overall comments: |  |  |  |  |
| 07 | Windows & lighting | Yes | No | N/A |
|  | Do windows open into rooms and not out onto corridors or walkways |  |  |  |
|  | Are windows protected to prevent children from falling out |  |  |  |
|  | Is any natural or electric lighting enough to enable children to clearly see from all areas of the room |  |  |  |
|  | Are teaching spaces (including green/white / chalkboards) free from glare or strong direct lighting |  |  |  |
| Overall comments: |  |  |  |  |
| 08 | Classroom layout | Yes | No | N/A |
|  | Is there a minimum space between desks or tables of 1500mmx1500mm |  |  |  |
|  | Does the room have detachable/moveable chairs and desks |  |  |  |
|  | Are there contrasting colours between the floor, desks/tables and walls |  |  |  |
|  | Is the lower end of any green / white / chalkboards 500mm from the floor |  |  |  |
|  | Does the floor of each room have a smooth, levelled, non-slip surface |  |  |  |
|  | Is the room generally free from obstructions, ordered and tidy |  |  |  |
|  | Does each class have a safe designated quiet area for children to spend short periods of time in |  |  |  |
| Overall comments: |  |  |  |  |
| 09 | Drinking water | Yes | No | N/A |
|  | Are areas designated for drinking water, level (step free), non-slip with good drainage |  |  |  |
|  | Is accessible drinking water available with one tap at a height of 400mm from the level surface |  |  |  |
|  | Do taps have lever handles |  |  |  |
| Overall comments: |  |  |  |  |
| 10 | Washroom facilities | Yes | No | N/A |
|  | Are accessible cubicles available in each main toilet block (e.g. they are not stand alone / separate from the main washrooms) |  |  |  |
|  | Are accessible toilet cubicles available for both boys and girls (e.g. one each in boys’ and girls’ toilets) |  |  |  |
|  | Is the door to the accessible cubicle a minimum of 900mm wide and open outwards |  |  |  |
|  | Is the size of the accessible toilet cubicle a minimum of 2000mm x 2200mm |  |  |  |
|  | Is there sufficient wheelchair manoeuvring space of 1500mm x 1500mm around the accessible toilet and washbasins |  |  |  |
|  | Are there grab rails each side of the accessible toilet at 455mm to 685mm from the floor surface |  |  |  |
|  | Is the toilet flush easy for a child to operate |  |  |  |
|  | Are toilet paper holders easy for children to reach from a sitting position |  |  |  |
|  | Is there a sanitary bin provided in all accessible female / unisex cubicles located on the wall side of the toilet |  |  |  |
|  | Is there a wash basin inside the accessible cubicle provided at 800-830mm from floor surface |  |  |  |
|  | Are doors easy to lock/unlock from the inside |  |  |  |
|  | Can entry be gained in an emergency |  |  |  |
|  | Are cubicles well lit (natural or electric light) |  |  |  |
|  | Are walls and doors high enough to prevent being seen from the outside |  |  |  |
|  | Are all floor surfaces inside the toilet blocks smooth, levelled, continuous and non-slip |  |  |  |
|  | Are all toilet blocks clean |  |  |  |
|  | Are toilet blocks located within sight of main school buildings |  |  |  |
| Overall comments: |  |  |  |  |
| 11 | Changing room for children managing toileting | Yes | No | N/A |
|  | Is there a space provided which is clean, dignified and wide enough to change a primary aged child lying down |  |  |  |
|  | Does it have a convenient shelf for placing items onto |  |  |  |
|  | Does it have fresh water for handwashing |  |  |  |
|  | Does it have a large trash disposal option with lid (to prevent flies) |  |  |  |
| Overall comments: |  |  |  |  |
| 12 | Menstrual hygiene | Yes | No | N/A |
|  | Do female / unisex cubicles have a small shelf, trash bin (with lid) and hook for changing clothes |  |  |  |
|  | Are handwashing options available inside the female / unisex toilet block or cubicles |  |  |  |
|  | If sanitary bins are provided, do they have lids |  |  |  |
|  | Are they emptied everyday |  |  |  |
|  | Is there a safe final disposal procedure in place |  |  |  |
| Overall comments: |  |  |  |  |
| 13 | Handwashing | Yes | No | N/A |
|  | Are handwashing facilities available within 10m of toilet blocks |  |  |  |
|  | Are areas designated for handwashing level (step free), non-slip with good drainage |  |  |  |
|  | Are children of all ages and height able to operate taps independently |  |  |  |
|  | Is soap freely available throughout the day |  |  |  |
|  | Are children of all ages and height able to reach the soap independently |  |  |  |
| Overall comments: |  |  |  |  |
| 14 | Playground | Yes | No | N/A |
|  | Is the surface level, smooth and clear of obstructions and obstacles |  |  |  |
|  | Does the playground have a paved pathway with a minimum width of 1800mm, along which a wheelchair can run |  |  |  |
|  | Is the play area fenced off from the public |  |  |  |
|  | Is the play area clear of hidden spaces |  |  |  |
|  | Is the play area clear of hazards (e.g. holes, ditches, uncovered drains, glass / metal) |  |  |  |
|  | Are there seating options available around play areas |  |  |  |
|  | Are there play areas available which are protected from direct sunlight |  |  |  |
|  | Are there options available for play from a wheelchair (e.g. raised sandpit, raised bed garden) |  |  |  |
| Overall comments: |  |  |  |  |
| 15 | Emergencies | Yes | No | N/A |
|  | Are emergency exits clearly marked with directional arrows |  |  |  |
|  | Are there both audio and visual emergency alarms in all areas |  |  |  |
|  | Are emergency exits accessible to wheelchair users |  |  |  |
|  | Does the school have an evacuation plan for students with disabilities |  |  |  |
| Overall comments: |  |  |  |  |